ESP32 Subcribe to NodeRed Message

Node red setup in browser

- Install Node Red (skip this if u already done install)

  - Install node js

  - Run command prompt and install Node Red

  - Run Node Red local server (do not close during using node-red)

  - Install palette dashboard @flowfuse/node-red-dashboard

- Open Node Red on browser

- Setup nodes as shown below

ESP32 , add 1 more LED

- Modify the nodes for Task 1 in Node Red

- Add the following nodes

- Setup MQTT OUT

- Setup Button ON led 1

- Setup Button OFF led 1

- Setup Button ON led 2

- Setup Button OFF led 2

- After Setup is done,click deploy

- Open dashboard

- The deployed dashboard in browser, is shown below.

- Split and Arrange Buttons into Group

- Go to Right Tab, Click Dashboard 2.0, Layout, Pages

- Click Page 1 , click + symbol (Add Group)

- Rename LED 2

- You can rename,delete  and set paramaeter by clicking gar button for each Group

- Arrange the button for each LED Group

- To Arrange you can use Drag and Drop or Double Click on Node, and assign manually

- Deploy again, after make any changes

- The deployed dashboard in browser, is shown below.

Circuit Setup using Hardware

- Connect the Hardware

- Refer the schematic diagram provided

- Check polarity and resistor value to make all will function properly

- Continue the setup for Arduino IDE

Setup codes in Arduino IDE

- Open Arduino IDE

- Copy code from this link : https://github.com/zulkeflipolimas/DEC30252-Student/blob/main/MQTT%20PreLab/mqtt_sub_2_LED.ino

- Modify the blank spot with suitable credential

  - Wifi SSID

  - Wifi Password

  - Broker Server (refer Node Red broker server link in MQTT OUT Nodes)

  - Subscribe Topic (refer Node Red topic in MQTT OUT Nodes)

- Modify MQTT Callback mqttCallback() to setup incoming MQTT messages and controls LEDs based on received commands

- The message must be the same with the Payload setting for each button in Node Red. Each payload message is unique for each button to avoid conflict.

- After done setup wifi credential and the MQTT callback

- Upload the code to the ESP32 board. Wait for the upload to finish.

- Open the Serial Monitor in Arduino IDE.

- Set baud rate to 115200.

- Observe the ESP32 connecting to WiFi and the connecting to MQTT server

Testing Receiving and Processing MQTT message

- Test the ON button on Node Red deployed dashboard

  - Observe the ESP32 LED, the LED should turn ON

  - Observe the Serial Monitor:

    - The received topic and message should be displayed.

    - The LED status should be indicated as ON.

- Test the OFF button on Node Red deployed dashboard

  - Observe the ESP32 LED, the LED should turn OFF.

  - Observe the Serial Monitor:

    - The received topic and message should be displayed.

    - The LED status should be indicated as OFF.

Publish LED Status to Node Red

- Add UI LED for the second LED

- Setup UI LED 1

- Setup UI LED 2

- Arrange, Label and deploy

- Your deployed page will look like this

Setup codes in Arduino IDE

- Open Arduino IDE

- Copy code from this link : https://github.com/zulkeflipolimas/DEC30252-Student/blob/main/MQTT%20PreLab/mqtt_sub_pub_2_LED.ino

- Modify the blank spot with suitable credential

  - Wifi SSID

  - Wifi Password

  - Broker Server (refer Node Red broker server link in MQTT OUT Nodes)

  - Subscribe Topic (refer Node Red topic in MQTT OUT Nodes)

  - Publish Topic (refer Node Red topic in MQTT IN Nodes)

- Upload the code to the ESP32 board. Wait for the upload to finish.

- Open the Serial Monitor in Arduino IDE.

- Set baud rate to 115200.

- Observe the ESP32 connecting to WiFi and the connecting to MQTT server

- Observe the LED on ESP32 and Browser.

Receiving and Processing MQTT message

- Test the ON OFF button LED1  on Node Red deployed dashboard

  - Observe the ESP32 LED, the LED should turn ON

  - Observe UI LED on dashboard browser LED1

  - Observe the Serial Monitor:

    - The received topic and message should be displayed.

    - The LED status should be indicated as ON.

- Test the ON OFF button LED2 OFF button on Node Red deployed dashboard

  - Observe the ESP32 LED, the LED should turn OFF.

  - Observe UI LED on dashboard browser LED2

  - Observe the Serial Monitor:

    - The received topic and message should be displayed.

    - The LED status should be indicated as OFF.

- Verify the message payload receive by open Debug Window

  - Make sure when button pressed, the payload on or off is appear on debug window as shown below

  - Debug window can help u tracing the flow of data, payload and message during your code is running.

  - Its include time and date when payload/message received